- A protocol defines what / how / when data is communicated

- The internet API is a set of rules that a sending program must follow so that Internet can deliver the data to the destination

- Networks can be classified on the types of transmission as circuit switching and packet switching

- OSI = open system interconnection

- OSI layers: application, presentation, session, transport, network, data link, physical

- TCP/IP layers:

  - Application = application + presentation + session from OSI

  - Transport = transport

  - Internet = network

  - Link = data link + physical

- Services of the data link layer: framing and link access, flow control, error correction

- O placa de retea poate avea mai multe adrese IP; un calculator poate avea mai multe placi de retea si mai multe IP-uri; placa de retea poate fi si externa; pentru conectarea unui calculator la o retea LAN se foloseste placa de retea

- MAC addresses - physically embedded into the network card, guaranteed to be unique; made of 6 bytes; MAC = media access control; in the data link layer; it can be changed

- Broadcast MAC address = FF.FF.FF.FF.FF.FF = logical address used to identify all the computers within a network

- A switch keeps a record of the MAC addresses of all the devices connected to it

- A computer can have multiple IP addresses

- 127.0.0.1 = localhost cannot be a network address nor a broadcast addr; it can be set as default gateway but not as DNS

- 0.0.0.0 is a valid netmask

- To find the MAC address on windows - ipconfig /all; to check if the data link layer works(?) - arp /a

- The optical fiber cable theoretically has unlimited bandwidth; but there is a limit to how much the end devices can send and receive + the ISP contract limits the bandwidth

- The bandwidth is the physical property of the transmission medium, throughput represents the amount of data which we transmit over a quantity of time

- Optical fiber -> light waves; wireless -> electro-magnetic waves

- Routing is done at the network level, is based on the destination IP - there is no routing based on MAC addresses

- IPv6 = 16 bytes, IPv4 = 4 bytes

- In class-full addressing - 5 addr classes

  - A - network part = 1 byte, host = 3 bytes; 1.0.0.0 -> 127.255.255.255; 2^7? networks (most significant bit is fixed => only 7 left)

  - B - network = 2 bytes, host = 2 bytes; 128.0.0.0 -> 191.255.255.255; 2^14 networks (first 2 bits are fixed..)

  - C - network = 3 bytes, host = 1 bytes; 192.0.0.0 -> 223.255.255.255; 2^21 networks (first 3 bits..)

  - D; 224.0.0.0 -> 239.255.255.255; used for multicast

  - E; 240.0.0.0 -> 255.255.255.255; never used

    - On some implementations, 255.255.255.255 is not part of class E, it’s the universal broadcast

  - Natural mask = masks of A, B, C

- Class-full addressing => size of the largest routing table =  2^7 + 2^14 + 2^21

- CIDR = Classless InterDomain Routing

  - Network part (= prefix) + host part, but the sizes are no longer multiples of 8bits

  - Host part = “/x”, x = number of bits in the host part

    - /24 ⇔ class C

    - /16 ⇔ class B

    - /8 ⇔ class A

  - The number of hosts is always a power of 2 (this includes the network addr + broadcast)

  - The beginning host address in each block must be divisible by the no. of hosts in that block; ex. a block of 16 addresses cannot start at a.b.c.36, but can start at a.b.c.64, or 48 etc.

- A network addr is correct if by ANDing it with the mask, we obtain the net addr; conversely, if we AND a random IP addr with its mask, we get its network addr

- To get the broadcast addr from the network -> OR it with the negated mask

- Supernetting = multiple routing entries become a single one (so the opposite of subnetting)

  - Ex: a.b.c.0 / 30 and a.b.c.4 / 30 can be seen as a.b.c.0 / 29

- A router has as many interfaces as the number of networks it connects

- In a routing table, if the gateway = 0 => the packet doesn’t need to be sent to another network, its destination network can be directly accessed by the router

  - If the destination and the mask = 0 => all addresses that don’t match any other entries are “caught” here

- Routing table can also be named forwarding table; it contains interface, netmask, destination, address, gateway

- All the hosts from the same network can physically reach each other without an intervening router

- Metric = the no. of routers that have to be passed in order to reach the destination; 1 actually means that no routers are passed (0 has another meaning)

- Private networks:

  - 10.0.0.0 / 8 (10.0.0.0 -> 10.255.255.255)

  - 172.16.0.0 / 12 (172.16.0.0 -> 172.31.255.255)

  - 192.168.0.0 / 16 (192.168.0.0 -> 192.168.255.255)

- Proxy server = intermediary for requests from clients seeking resources from other clients

- DHCP = dynamic host configuration protocol

- A DHCP server can relay IP addr to clients on a network segment separated from the server’s location when the router that separates them acts as a relay agent

- It’s possible to have +1 DHCP server on the same subnet if each has its own, disjoint pool of addresses

- DHCP uses UDP at the transport layer

- CLI = command line interface

- SCTP = Stream Control Transmission Protocol; in the transport layer

- DSL = digital subscriber line; used to give access to internet through telephone lines

- Crossover cable: connect switch -> switch, switch -> hub, hub -> hub, router -> router, router -> pc, pc -> pc

- Straight through: connect router -> switch, switch -> pc / server, hub -> pc / server

  - Obviously the reverse work as well

- The switch sends a packet specifically to an endpoint (or more), the hub broadcasts the message to all the network

- A switch can transport UDP / IP / TCP packets

- A hub does not understand a MAC address; a switch does

- Big endian = most significant byte first; little endian = least …

  - The network standard is big endian, that’s why we use ntoh and hton

- 2 PCs might not be able to ping each other if both firewalls are enabled

- Gateways are used for providing connectivity between +2 network segments

- A computer cannot have 2 gateways

- RIP = routing information protocol

- RIPv1 doesn’t support classless routing protocols (but v2 does); it has the same timers as v2

- Maximum metric of RIPv2 = 15; any further is considered unreachable

- Float uses 4 bytes in 32bit systems; 8 in 64bit systems

- SSH = secure shell; used for remote connection to a terminal / command line; in the application layer

- P2P = peer to peer

- Exchange units at each layer:

  - App layer - data structure

  - TCP - segment, UDP - datagram

  - IP - datagram / packet

  - MAC layer - frame

- IP protocol = best effort; doesn’t guarantee successful delivery, unicity, data integrity

- IP Datagram header = 20 bytes

  - Version - 4 bits, usually 4 / 6, if it’s ipv4, ipv6

  - Header length - 4 bits; it counts how many 32-bit entities there are

  - Type of service - 8 bits; type of content (file, voice, video..), related to priorities

  - Length - 16 bits; of the entire datagram in bytes => max IP datagram size = 64kb

  - 16-bit identifier; given to each datagram that leaves the host

  - Flags - 3 bits

    - DF = “don’t fragment”; if it’s set to 1 and a packet doesn’t fit => the packet is not sent, the sender is notified

    - MF = “more fragments”, set to 1 whenever a packet is split

  - 13-bit fragment offset; if a packet doesn’t fit on a connection, it will be fragmented and it won’t be reassembled until it reaches the destination

    - The split packets all have the same 16bit ID

    - Beware of the 20 byte header size

  - TTL = time to live - 8 bits; = nr. of routers that the datagram can pass before it’s discarded; decremented when passing through a router, when it reaches 0 => dg is discarded and a signal is given (TTL expired) to the host of this dg

  - Upper layer - 8 bits; which protocols are transported inside this dg (ex. TCP = 6, UDP = 17)

  - Header internet checksum - 16 bits; = the 16bit one’s complement of the one’s complement sum of all 16bit words in the header; while computing this, we consider the value of the checksum inside the header to be 0

  - Source IP - 32 bits

  - Destination IP - 32 bits

- MTU = maximum transfer unit (through a connection)

- ARP = address resolution protocol; basically determines the destination MAC address given its IP; it’s not used for determining the IP when the MAC is known; uses broadcast

- NAT = network address translation

  - About 64k simultaneous connections for TCP and 64k for UDP with a single LAN address (bc the port-number field is 16bits)

  - The outside world only sees 1 IP addr

  - Port forwarding is an application of it

  - Increases flexibility when connecting to the internet

  - Causes loss of end-to-end IP traceability

  - Reduces address overlap occurrence

- ICMP = internet control messaging protocol

  - Signaling protocol - error reporting, pings

    - Dest network unreachable - the router cannot route to a network, it has no idea about it

    - Dest host unreachable - the (final) destination cannot be reached, maybe it’s protected by firewall or sth

    - Dest protocol unreachable - an IP connection can be established, but not a TCP / UDP (perhaps a firewall again)

    - Bad ip header - checksum doesn’t correspond

  - its data is mostly useless, it's there to ensure that there's been no modifications to the packet in transit

  - Encapsulated within IP datagrams

- Traceroute

  - is a diagnostic tool for displaying the path and measuring transit delays of packets across an IP network

  - It first sends a packet (usually ICMP) with TTL = 1 => we get the first router, TTL = 2 => second … and so on

- UDP = user datagram protocol

  - Process to process comunication

  - Header = 8 bytes (just for it, there are also 20 bytes from IP, and others from the app layer)

    - Source port + dest port (16 bits each)

    - Length = 16 bits, of the entire datagram

    - Checksum = 16 bits, computed over the header + data of UDP + IP

  - The datagrams’ integrity is only checked when reaching the (final) destination

  - No congestion control (as opposed to TCP), so it can overflow

  - Datagram delivery is not guaranteed

  - Uses the socket SOCK_DGRAM

- TCP = transmission control protocol

  - Ordered data transfer, retransmission of lost packets => error-free

  - It has flow control (as opposed to UDP), so no party can overflow the other; traffic is controlled by the OS

  - It writes to a stream of bytes, while UDP writes packets

  - Uses the socket SOCK_STREAM

  - Header = 20 bytes (+ the 20 from IP and others from the app layer)

    - Source port + dest port (16 bits each)

    - Sequence number = 32bits, counts the amount of bytes exchanged over that connection

    - Acknowledgement number = 32bits, the index of the next expected byte

    - Header length = 4bits, it counts how many 32bit entities are there; also called data offset

    - Flags = 6 bits

      - ACK; 1 if the acknowledgement number is ok

      - SYN - synchronize when creating the connection

      - FIN - final, when closing a connection

    - Window size = 16 bits, for flow control (how much space left in the buffer of the receiver)

    - Checksum = 16bits, same algorithm as IP, also ‘uses’ header + data from IP and TCP

    - Urgent pointer = 16bits, not frequently used

  - When initialised, a TCP connection needs a state, at the kernel level for both sender / receiver

    - Starting sequence number (sent)

    - Received sequence number

    - 2 buffers (queues), one for sending, one for receiving

  - Receiver window -

  - A segment is retransmitted if a timer expires; the timer is changed according to previous results

    - RTT = round-trip delay time;

    - estimatedRTT = (1 - a) * previous_estiamatedRTT + a * sampleRTT; a = 0.125 usually

    - devRTT = (1 - b) * previous_devRTT + b * abs(sampleRTT - estimatedRTT)

    - Timeout interval = estimatedRTT + 4 * devRTT

  - Congestion = too much data is sent too fast for the network to handle; symptoms = lost packets, long delays

    - Network-assisted congestion control = the router provides info about the current situation to its end points, rarely used

    - End-end congestion control - the congestion status is inferred

    - Congestion window - starts from 1; when below a threshold => grows exponentially (slow-start phase); when above => grows linearly (congestion-avoidance); it’s imposed by the sender

    - Triple duplicate ACK occurs => threshold = cw / 2, cw = threshold

    - Timeout occurs => threshold = cw / 2, cw = 1

  - Fairness: k TCP sessions => each gets R / K of the bandwidth

- connect() is normally called by the client in order to connect to a server

- bind() is not mandatory for the client when establishing a TCP connection (when the socket is created, is gets a random IP + port by default, so it works even if we don't specify them ourselves);

| Mandatory calls | TCP | UDP |
| --- | --- | --- |
| client | connect(), socket() | socket() |
| server | accept(), listen(), bind(), socket() | bind(), socket() |

- Application layer protocols: DNS, SMTP, FTP, SSH

- DNS = domain name system

  - A distributed database of domain names and IPs

  - Uses UDP, port 53

  - TLD = top level domain (.com, .ro, …); ROTLD = Romanian TLD; subdomain = www.plm.cs.ubbcluj.com

  - FQDN = fully qualified domain name = hostname + domain name (www.site.com)

  - Root DNS servers = global, serve domains like .com, .org etc.; about 16-64 of them; they are known by all the OS which allow DNS

  - Resolver = the software component that translates IP to hostname

  - Iterative website query: local DNS server -> a root DNS s -> local DNS -> the DNS s for the domain obtained above -> local DNS -> the DNS s of the website -sends its IP to-> local DNS -> access the website by its IP

  - Recursive website query: instead of returning info back to the local DNS at every step, that server goes directly to the next one (ex. from root DNS to the domain DNS)

    - Not frequent in practice, bc those servers basically do the work of our local DNS s

- A DNS server can be a default gateway

- The dot in www.site.com is unrelated to the dot in an IP address

- ICANN = Internet Corporation for Assigned Names and Numbers - supervises all the domain registrars = companies that register domain names (ex. rnc.ro)

- Whois - query used for checking if a domain name is already bought

- It’s recommended that we have at least 2 DNS servers for a domain name, in case one fails (if possible not in the same location, bc earthquakes / fires / tzanca uraganu)

  - There’s a primary / master one, the others are secondary / slaves and are periodically synchronized with the master

- Resource records = (domain name, TTL, class, type, value);

  - Types:

    - A = hostname for an IP add

    - NS =  domain

    - CNAME = alias for (canonical) real name

    - MX = name of mailserver for that domain

    - SOA = start of authority

- Query DNS packet = QName (hostname) + QType (A, MX…) + QClass (type of addressing, IN = internet)

- FTP = file transfer protocol

  - Exchange files between 2 machines

  - It’s a text protocol

  - 2 modes: active and passive

  - SMB or SAMBA protocols

  - 2 channels - control (command) and data

  - Not encrypted

- SMTP = simple mail transfer protocol

  - Text protocol, implemented by MX servers

  - Uses TCP

  - Works on port = 25

  - Allows for offline msg exchanging

  - When configuring email clients, an Internet address for an SMTP server must be entered

  - Used for reading mails:

    - POP3 = post office protocol

    - IMAP = internet mail access protocol

- HTTP = hypertext transfer protocol

  - Works on port = 80

  - Is human readable

  - Also involved in sending email